|  | Thermal Survey Report 2300 Area Pipework123 |  |  |
| --- | --- | --- | --- |

Contents

1	General Information	3

2	Defect Overview	3

3	Figures	4

Document information

| Project name | Thermography | Document location | G:// Drive | Document type | Report |
| --- | --- | --- | --- | --- | --- |
|  | Function | Name | Department | Signature | Date |
| Peer review |  |  |  |  |  |
| Approval |  |  |  |  |  |
| Document history | Version | Date | Name | Description |  |
|  | 1 | 17/08/2026 | Sam Drawwater | 2300 Area Pipework Thermography report |  |

# General Information

| Thermographer | Sam Drawwater |
| --- | --- |
| Thermographer e-mail | sam.drawwater@glencore.com.au |
| Inspection scope | Grinding Area Pipework |
| Inspection period | 05/08/2026 – 08/08/2026 |
| Camera producer | FOTRIC |
| Camera model | P5 MIX |
| Serial number | 0202005880 |
| Operating conditions during the survey | Normal conditions |

| Rating | Temp Differential ˚C |  | Recommendation |
| --- | --- | --- | --- |
| Okay | 0 | 1.5 | No action required. |
| Attention | 1.5 | 2.0 | Monitor further. |
| Severe | 2.0 | 2.5 | Schedule in next 1 to 2 shutdowns. |
| Critical | >2.5 |  | Schedule for next shutdown period. |

# Defect Overview

| Overview of defects divided into defect categories |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 117 | 27 | 13 | 18 |  |

| Description of Defect Categories |  |  |
| --- | --- | --- |
| Not Collected |  | Point missed or not collectable |
| Okay |  | No action required |
| Attention |  | Possible defect requires monitoring |
| Severe |  | Defect that needs action in ≤8 months |
| Critical |  | Defect that needs action ≤4 months |

| Inspection Point | Area | Asset | Status | Note | Notification/Work Order |
| --- | --- | --- | --- | --- | --- |
| 1 | 2300 | 2300PI001 | Severe | Needs to be replaced when 2300SN001 is replaced. 2300PU002 bellows is over stretched and needs replacing | W/O: 10721418 Noti: 101765431 |
| 7 | 2300 | 2300PI001 | Attention | Monitor |  |
| 8 | 2300 | 2300PI001 | Severe | Replace in March 2027 shutdown | Noti: 101765439 |
| 10 | 2300 | 2300PI001 | Critical | Replace in November 2026 shutdown | Noti: 101765441 |
| 11 | 2300 | 2300PI001 | OK | Monitor |  |
| 12 | 2300 | 2300PI001 | OK | Monitor |  |
| 14 | 2300 | 2300PI001 | Attention | Monitor |  |
| 15 | 2300 | 2300PI001 | OK | Monitor |  |
| 29 | 2300 | 2300PI001 | OK | Monitor |  |
| 29 | 2300 | 2300PI001 | OK | Monitor |  |
| 30 | 2300 | 2300PI001 | Attention | Monitor |  |
| 31 | 2300 | 2300PI001 | Critical | Replace in November 2026 shutdown | Noti: 101765429 |
| 33 | 2300 | 2300PI001 | OK | Monitor |  |
| 34 | 2300 | 2300PI001 | OK | Monitor |  |
| 38 | 2300 | 2300PI001 | Critical | Replace in November 2026 shutdown | Noti: 101765460 |
| 1 | 2300 | 2300PI002 | Severe | Monitor – Related to Turbulence |  |
| 4 | 2300 | 2300PI002 | Severe | 2300PU031 Suction showing signs of wear across Straub clamp | Noti: 101765448 |
| 6 | 2300 | 2300PI002 | Critical | 2300PU031 signs of wear in second discharge spool | Noti: 101765480 |
| 7 | 2300 | 2300PI002 | Critical | 230PU031 signs of wear near discharge flange on mine hose bend | Noti: 101770528 |
| 8 | 2300 | 2300PI002 | Attention | Signs of build-up inspect when mine hose replaced |  |
| 9 | 2300 | 2300PI002 | Critical | 2300PU031 Signs of wear or delamination near feed flange and external age deterioration | Noti: 101770533 |

| Inspection Point | Area | Asset | Status | Note | Notification/Work Order |
| --- | --- | --- | --- | --- | --- |
| 10 | 2300 | 2300PI002 | Critical | 2300PU031 Damage or delamination of lining near feed side flange related to misalignment or mismatched internal diameters | Noti: 101770529 |
| 11 | 2300 | 2300PI002 | Attention | Signs of turbulence. Monitor |  |
| 12 | 2300 | 2300PI002 | Attention | Signs of slight build up. Monitor |  |
| 16 | 2300 | 2300PI002 | Severe | Signs of wear near discharge side flange related to incorrect length mine hose fitted causing stretch across flange | Noti: 101770541 |
| 21 | 2300 | 2300PI002 | Critical | Signs of damage at the discharge side flange potentially due to the mine hose being pulled out of align. | Noti: 101770517 |
| 22 | 2300 | 2300PI002 | Critical | Signs of wear on feed side flange most likely related to misalignment or damage in the mine hose connected to it. | Noti: 101770518 |
| 26 | 2300 | 2300PI002 | Severe | Due for replacement in November | WO: 10718808 |
| 2 | 2300 | 2300PI003 | Attention | Monitor. Possible signs of wear |  |
| 3 | 2300 | 2300PI003 | Critical | Signs of wear near the feed side flange caused by the bend being too short for the location it is installed in. | Noti: 101770561 |
| 4 | 2300 | 2300PI003 | Severe | Monitor. Signs of turbulence likely related to mine hose defect above |  |
| 5 | 2300 | 2300PI003 | Attention | Possible signs of wear or build up. Monitor. |  |
| 6 | 2300 | 2300PI003 | Attention | Possible signs of wear or build up. Monitor. |  |
| 7 | 2300 | 2300PI003 | Attention | Possible signs of wear or build up. Monitor. |  |
| 8 | 2300 | 2300PI003 | Attention | Possible signs of wear or build up. Monitor. |  |
| 9 | 2300 | 2300PI003 | Attention | Possible signs of wear or build up. Monitor. |  |
| 10 | 2300 | 2300PI003 | Attention | Possible signs of wear or build up. Monitor. |  |

| Inspection Point | Area | Asset | Status | Note | Notification/Work Order |
| --- | --- | --- | --- | --- | --- |
| 12 | 2300 | 2300PI003 | Attention | Possible signs of wear or build up. Monitor. |  |
| 14 | 2300 | 2300PI003 | Severe | Possible signs of wear. Replace the mine hose. | Noti: 101770549 |
| 1 | 2300 | 2300PI004 | Severe | 2300SU005 Signs of wear in discharge spigots | Noti: 101770570 |
| 4 | 2300 | 2300PI004 | Attention | Signs of turbulence. Monitor |  |
| 7 | 2300 | 2300PI004 | Attention | Signs of turbulence. Monitor |  |
| 11 | 2300 | 2300PI004 | Attention | Signs of turbulence or build up. Monitor |  |
| 12 | 2300 | 2300PI004 | Attention | Signs of turbulence. Monitor |  |
| 13 | 2300 | 2300PI004 | Attention | Signs of build-up. Monitor |  |
| 16 | 2300 | 2300PI004 | Attention | Signs of turbulence. Monitor |  |
| 4 | 2300 | 2300PI005 | Attention | Early signs of wear or turbulence at the discharge of the Sag Mill Dissipater. Monitor. |  |
| 12 | 2300 | 2300PI005 | Critical | Previously reported signs of wear have worsened. Replace in November 2026 shut | Noti: 101770572 Previous WO: 10507154 |
| 1 | 2300 | 2300PI006 | Severe | Dissipator discharge repaired in July 2026 shut. Still showing signs of wear, needs replacing in future shut. | Noti: 101770555 |
| 2 | 2300 | 2300PI006 | Attention | Pipe looks to be bogged with epigen on outside and wrapped in plastic. Replace. | Noti: 101770575 |
| 3 | 2300 | 2300PI006 | Attention | Possible signs of build -up. Monitor |  |
| 6 | 2300 | 2300PI006 | Severe | Signs of wear at feed side flange related to misalignment or mismatched internal diameters. | Noti: 101770557 |
| 8 | 2300 | 2300PI006 | Attention | Wear or build up in weirs. Monitor |  |
| 5 | 2300 | 2300PI007 | Critical | Wear in multiple places | Noti: 101770567 |
| 7 | 2300 | 2300PI007 | Critical | Signs of wear in multiple places | Noti: 101770559 |
| Inspection Point | Area | Asset | Status | Note | Notification/Work Order |
| 10 | 2300 | 2300PI007 | Critical | Replace in November 2026 | WO: 10714117 |
| 25 | 2300 | 2300PI007 | Critical | Signs of rubber lining damage throughout spool | Noti: 101770569 |
| 30 | 2300 | 2300PI007 | Attention | Hot spot related to wear or build up. Monitor |  |
| 33 | 2300 | 2300PI007 | Severe | Signs of build up. Section needs cleaning. | Noti: 101770600 |
| 36 | 2300 | 2300PI007 | Severe | Bend wrapped for a long time and showing further signs of lining deterioration. | Noti: 101770584 |
| 42 | 2300 | 2300PI007 | Attention | Possible signs of wear related to turbulence. Monitor. |  |
| 43 | 2300 | 2300PI007 | Critical | 2500BX003 Plant feed box multiple signs of worn lining | Noti: 101770594 |
| 45 | 2300 | 2300PI007 | Critical | 2500SA002 multiple signs of wear to lining | Noti: 101770595 |

# Detailed Analysis

| Inspection Point | 1 | Asset | 2300PI001 |  | Notification Number |  |
| --- | --- | --- | --- | --- | --- | --- |
| Differential Temp. |  |  |  |  | Inspection Status |  |
| Text Annotation: |  |  |  |  |  |  |